# MEASURING BLADDER PRESSURE

Training manual for nurses

© Child-Help International & SHARE Knowledge Centre

## Purpose of measuring bladder pressure

## Evaluate the effect of the Oxybutynin treatment

## Check if the full bladder has a safe pressure = below 30cm H2O water

## NOTE: in case of a bladder infection, bladder measurement is not useful because the bladder muscle reacts to the infection

## Two Simple techniques

## Fill bladder with normal saline solution to a maximum of 5 to 10ml/min

## Measure with feeding tube after natural filling

## Give a clear explanation to mother and child

## Put the child in a comfortable lying position

# Technique 1: Filling of bladder with normal saline solution of 0,9%

## Necessary materials:

## Normal saline solution

## Three-way tap

## Infuse set

## Catheter

## Measuring tube (water manometer)

## Collecting bowl

## Disinfectant

## Compress

## Technique:

## Collect the necessary materials

## Place the catheter in the bladder, empty the bladder and record the catheterized volume

## Connect the three-way tap to the catheter

## Connect the measuring tube and the infuse to the tap

## Hold the tubes at the level of the urethra = same height as bladder entrance

## Close the three-way crane at measuring tube

## Let normal saline slowly run at a maximum of 5-10ml /min

## If there is urine loss outside the catheter: stop the saline infusion and open the three-way tap at measuring tube

## The numbers on the measuring tube indicate the bladder pressure

## Catheterize the bladder of the patient after examination and record the quantity

## Technique 2: Measuring natural bladder filling with feeding tube

## Necessary materials:

## Catheter CH 8

## Collection bowl

## Measuring tape

## Technique:

## Place and secure the catheter

## Close off the catheter and secure on the belly

## If there is leaking along the catheter, keep the catheter upright and open at the top

## Place measuring tape on pubic bone (= +/- height bladder entrance)

## Hold the catheter alongside the measuring tape

## Read the bladder pressure

## Catheterize bladder and measure quantity

## After catheterization of the bladder, the basic pressure can be measured